___________________________________________________________________________

# IA621001 Automation and Robotics

In-class practicals - Ballistic Behaviour SBA

___________________________________________________________________________

| Study Block: | Block 1, 2026 |
| --- | --- |
| Course Leader: | Dr Senaka Amarakeerthi |
| Due date and time: | 27th Februar 2026 – 10.30AM to 12.30PM |
| Submission: | Moodle Submission and a demonstration. |
| Submission format: | Word document with all code snippets. |
| Weighting/Contribution: | 10% |
| Marks out of | 30 |
| Course Level | 6 |
| Credits | 15 |
| Learning outcomes covered: | LO 3 |
| Pass Criteria | Cumulative 50% pass |
| Version | 1.0 |

# Learning Outcomes Covered
LO3: Use an appropriate software development platform to implement simple interactive robotics/automated systems.

# General Description and Conditions

- This assessment task is designed to show you can apply core principles to robot design, design and implement a robotic solution to a problem

- The result of this task will accrue towards your final mark for this paper.

- SOFTWARE:

  - Software must be your own work and be uploaded to Moodle before the end of the session. In this early stage, I’m looking for performance from your robot rather than perfect code but try to make your code as neat and tidy as you can. Keep Loop() nice and clean as per Embedded Systems.

- You will not be expected to account for the general state of the room, conditions on the ground, or unexpected conditions in the test area. For example, if your robot snags on the carpet or furniture.

- This SBA is open-book. You may make use of any reference to help you debug this task.

- A viva voce examination will be conducted at the end of the demonstration. Failure to demonstrate your understanding and involvement in the code will result in an integrity case for further investigation.

Assessment Task

You will program your robot to demonstrate the following basic behaviours. The behaviours will be observable and assessed live.

## Square Spiral

Robot drives in straight lines and turns 90 degrees. Robot may spiral inward, or outward.

## Zig-zag

Robot drives in a zig-zag fashion. (Drive-turn-drive-turn-turn-drive, etc)

## Square zig-zag

Robot drives in a square zig-zag, to cover a square area (similar to mowing a lawn)

## Circle.(Not a spiral)

Robot drives in a circle. The end point of the circle should meet the start point (Otherwise we’re driving in a spiral)

APPENDIX: SOURCE CODE

================================================================================

FILE 1: ZumoOS.ino - Main Program Entry Point

================================================================================

/*

* ZumoOS.ino - Main Program Entry Point

*

* ========== ASSIGNMENT INFORMATION ==========

* Author: Bhanu

* Date: February 27, 2026

* Assignment: SBA - Ballistic Behaviors for Autonomous Robot

*

* ========== WHAT THIS FILE DOES ==========

* This is the MAIN file of the Zumo robot program. It's the entry point

* that Arduino runs when the robot powers on. This file coordinates all

* other components of the system.

*

* ========== PROGRAM STRUCTURE ==========

* Arduino programs have two required functions:

* 1. setup() - Runs ONCE when robot powers on

* 2. loop() - Runs CONTINUOUSLY forever after setup()

*

* ========== WHAT THIS PROGRAM DOES ==========

* This program implements 5 ballistic behaviors for autonomous robot movement:

*

* ASSIGNMENT REQUIRED (4):

* 1. SQUARE SPIRAL - Square spiral with 90° turns (inward/outward)

* 2. ZIG-ZAG - Alternating diagonal movements

* 3. SQUARE ZIG-ZAG - Parallel vertical lines pattern

* 4. CIRCLE - Single continuous circular path

*

* ADDITIONAL (for comparison):

* 5. SPIRAL - Circular spiral (NOT in assignment, for comparison only)

*/

#include "Hardware.h"  // Hardware initialization and motor control functions

#include "Menu.h"      // User interface for mode selection

#include "Modes.h"     // Ballistic behavior implementations

#include "Config.h"    // Configuration constants (speeds, timings, etc.)

void setup() {

Serial.begin(9600);        // Initialize serial communication at 9600 baud

initializeHardware();      // Initialize motors, sensors, display, and IMU

playStartupSound();        // Play a sound to indicate successful startup

delay(500);                // Brief pause for system stabilization

}

void loop() {

Mode selectedMode = selectMode();  // Show menu and get user's mode choice

runMode(selectedMode);             // Execute the selected ballistic behavior

display.clear();                   // Clear the OLED display

display.print("Mode Done!");       // Show completion message

delay(1000);                       // Wait 1 second so user can see message

}

================================================================================

FILE 2: Config.h - Configuration Constants

================================================================================

/*

* Config.h - Configuration Constants

*

* This file contains all tunable parameters and constants for the robot.

*/

#pragma once

// ============ MOTOR SPEEDS ============

#define ENCODER_SPEED 250

// Speed for encoder-based movement

// Used in ballistic behaviors for straight lines

// ============ TIMING ============

#define MENU_TIMEOUT_MS 10000

// Menu auto-timeout (10 seconds)

#define BUTTON_DEBOUNCE_MS 50

// Button debounce delay (50 milliseconds)

// ============ ENCODER CONSTANTS ============

// NOTE: Values DOUBLED from original 320/640 because physical turns

// were measuring less than 90 degrees. Adjust based on testing.

#define TURN_90_TICKS 640

// Encoder ticks for 90-degree turn (DOUBLED for testing)

// Original value was 320, doubled to 640

#define TURN_180_TICKS 1280

// Encoder ticks for 180-degree turn (DOUBLED for testing)

// Original value was 640, doubled to 1280

#define SQUARE_SIDE_TICKS 900

// Encoder ticks for square side length

// ============ TURN SPEED ============

#define TURN_SPEED 250

// Speed when turning (used by turnRight90, turnLeft90, turn180)

// ============ CALIBRATION ============

#define CALIBRATION_SPINS 80

// Number of calibration readings to take

#define CALIBRATION_DELAY_MS 20

// Delay between calibration readings

================================================================================

FILE 3: Modes.h - Mode Definitions

================================================================================

/*

* Modes.h - Ballistic Behavior Mode Definitions

*/

#pragma once

enum Mode {

MODE_SPIRAL,          // Circular spiral (NOT in assignment - for comparison)

MODE_SQUARE_SPIRAL,   // Square spiral with 90° turns (REQUIRED)

MODE_ZIGZAG,          // Alternating diagonal movements (REQUIRED)

MODE_SQUARE_ZIGZAG,   // Parallel vertical lines (REQUIRED)

MODE_CIRCLE,          // Continuous circular path (REQUIRED)

MODE_COUNT            // Total number of modes

};

void runMode(Mode m);

// Ballistic behavior function declarations

void modeSpiral();

void modeSquareSpiral();

void modeZigzag();

void modeSquareZigzag();

void modeCircle();

================================================================================

FILE 4: Modes.cpp - Ballistic Behavior Implementations

================================================================================

#include "Modes.h"

#include "Hardware.h"

#include "Config.h"

#include "Utils.h"

void runMode(Mode m) {

switch(m) {

case MODE_SPIRAL: modeSpiral(); break;

case MODE_SQUARE_SPIRAL: modeSquareSpiral(); break;

case MODE_ZIGZAG: modeZigzag(); break;

case MODE_SQUARE_ZIGZAG: modeSquareZigzag(); break;

case MODE_CIRCLE: modeCircle(); break;

default: break;

}

}

// ============ SPIRAL (NOT IN ASSIGNMENT - FOR COMPARISON) ============

void modeSpiral() {

display.clear();

display.print("A:In C:Out");

display.gotoXY(0, 1);

display.print("B:Cancel");

bool directionSelected = false;

bool inward = true;

while (!directionSelected) {

if (buttonA.getSingleDebouncedPress()) {

inward = true;

directionSelected = true;

}

if (buttonC.getSingleDebouncedPress()) {

inward = false;

directionSelected = true;

}

if (buttonB.getSingleDebouncedPress()) {

return;

}

delay(50);

}

display.clear();

display.print(inward ? "Inward" : "Outward");

delay(1000);

int outerSpeed = 400;

int startInnerSpeed = inward ? 350 : 100;

int endInnerSpeed = inward ? 100 : 350;

float speedChangeRate = inward ? -0.5 : 0.5;

unsigned long loopDelay = 30;

float currentInnerSpeed = startInnerSpeed;

while (true) {

if (inward && currentInnerSpeed <= endInnerSpeed) break;

if (!inward && currentInnerSpeed >= endInnerSpeed) break;

int innerSpeed = (int)currentInnerSpeed;

if (innerSpeed < 50) innerSpeed = 50;

if (innerSpeed > 500) innerSpeed = 500;

motors.setSpeeds(innerSpeed, outerSpeed);

currentInnerSpeed += speedChangeRate;

delay(loopDelay);

if (buttonB.isPressed()) break;

}

stopMotors();

display.clear();

display.print("Complete!");

delay(1000);

}

// ============ SQUARE SPIRAL (REQUIRED) ============

void modeSquareSpiral() {

display.clear();

display.print("A:In C:Out");

display.gotoXY(0, 1);

display.print("B:Cancel");

bool directionSelected = false;

bool inward = true;

while (!directionSelected) {

if (buttonA.getSingleDebouncedPress()) {

inward = true;

directionSelected = true;

}

if (buttonC.getSingleDebouncedPress()) {

inward = false;

directionSelected = true;

}

if (buttonB.getSingleDebouncedPress()) {

return;

}

delay(50);

}

display.clear();

display.print(inward ? "Inward" : "Outward");

delay(1000);

int sideLength = inward ? 1400 : 200;

int increment = 70;

int speed = 500;

int minSide = 150;

int maxSide = 1500;

while (true) {

if (inward && sideLength < minSide) break;

if (!inward && sideLength > maxSide) break;

driveTicks(sideLength, speed);

turnRight90();

if (inward) {

sideLength -= increment;

} else {

sideLength += increment;

}

if (buttonB.isPressed()) break;

}

stopMotors();

display.clear();

display.print("Complete!");

delay(1000);

}

// ============ ZIG-ZAG (REQUIRED) ============

void modeZigzag() {

display.clear();

display.print("Starting...");

delay(500);

int segmentLength = 900;

int zigzags = 6;

int speed = 400;

int turnAngleTicks = 480;

for (int i = 0; i < zigzags; i++) {

driveTicks(segmentLength, speed);

delay(100);

if (i % 2 == 0) {

resetEncoders();

while (abs(encoders.getCountsLeft()) < turnAngleTicks) {

motors.setSpeeds(400, -400);

if (buttonB.isPressed()) break;

}

motors.setSpeeds(0, 0);

delay(100);

} else {

resetEncoders();

while (abs(encoders.getCountsLeft()) < turnAngleTicks * 2) {

motors.setSpeeds(-400, 400);

if (buttonB.isPressed()) break;

}

motors.setSpeeds(0, 0);

delay(100);

}

if (buttonB.isPressed()) break;

}

stopMotors();

display.clear();

display.print("Complete!");

delay(1000);

}

// ============ SQUARE ZIG-ZAG (REQUIRED) ============

void modeSquareZigzag() {

display.clear();

display.print("Starting...");

delay(500);

int columnHeight = 1400;

int columns = 3;

int columnSpacing = 400;

int speed = 350;

for (int i = 0; i < columns; i++) {

driveTicks(columnHeight, speed);

delay(150);

if (i < columns - 1) {

if (i % 2 == 0) {

turnRight90();

delay(150);

driveTicks(columnSpacing, speed);

delay(150);

turnRight90();

delay(150);

} else {

turnLeft90();

delay(150);

driveTicks(columnSpacing, speed);

delay(150);

turnLeft90();

delay(150);

}

}

if (buttonB.isPressed()) break;

}

stopMotors();

display.clear();

display.print("Complete!");

delay(1000);

}

// ============ CIRCLE (REQUIRED) ============

void modeCircle() {

display.clear();

display.print("Starting...");

delay(500);

int outerSpeed = 450;

int innerSpeed = 250;

unsigned long circleDuration = 7000;

unsigned long startTime = millis();

while (millis() - startTime < circleDuration) {

motors.setSpeeds(outerSpeed, innerSpeed);

if (buttonB.isPressed()) break;

}

stopMotors();

display.clear();

display.print("Complete!");

delay(1000);

}

================================================================================

FILE 5: Hardware.h - Hardware Interface Declarations

================================================================================

#pragma once

#include <Wire.h>

#include <Zumo32U4.h>

extern Zumo32U4Motors motors;

extern Zumo32U4Encoders encoders;

extern Zumo32U4LineSensors lineSensors;

extern Zumo32U4ProximitySensors prox;

extern Zumo32U4OLED display;

extern Zumo32U4Buzzer buzzer;

extern Zumo32U4ButtonA buttonA;

extern Zumo32U4ButtonB buttonB;

extern Zumo32U4ButtonC buttonC;

extern Zumo32U4IMU imu;

extern unsigned int lineValues[5];

extern int lastError;

void initializeHardware();

void playStartupSound();

void playModeChangeSound();

void playErrorSound();

void playSuccessSound();

int clampSpeed(int speed);

void stopMotors();

void setMotorSpeeds(int left, int right);

================================================================================

FILE 6: Hardware.cpp - Hardware Implementation

================================================================================

#include "Hardware.h"

#include "Config.h"

Zumo32U4Motors motors;

Zumo32U4Encoders encoders;

Zumo32U4LineSensors lineSensors;

Zumo32U4ProximitySensors prox;

Zumo32U4OLED display;

Zumo32U4Buzzer buzzer;

Zumo32U4ButtonA buttonA;

Zumo32U4ButtonB buttonB;

Zumo32U4ButtonC buttonC;

Zumo32U4IMU imu;

unsigned int lineValues[5];

int lastError = 0;

void initializeHardware() {

display.clear();

display.print("ZumoOS Pro");

display.gotoXY(0, 1);

display.print("Initializing");

Wire.begin();

if (imu.init()) {

imu.enableDefault();

}

delay(1000);

display.clear();

}

void playStartupSound() {

buzzer.play("c32 e32 g32 c64");

}

void playModeChangeSound() {

buzzer.play("g32 c32");

}

void playErrorSound() {

buzzer.play("c16 c16 c16");

}

void playSuccessSound() {

buzzer.play("g32 a32 b32 c64");

}

int clampSpeed(int speed) {

return speed;

}

void stopMotors() {

motors.setSpeeds(0, 0);

}

void setMotorSpeeds(int left, int right) {

motors.setSpeeds(left, right);

}

================================================================================

FILE 7: Utils.h - Utility Function Declarations

================================================================================

#pragma once

#include <Arduino.h>

void waitForButtonPress();

void calibrateLineSensors();

void calibrateProximitySensors();

void showSensorReadings();

void performSystemCheck();

int mapRange(int value, int fromLow, int fromHigh, int toLow, int toHigh);

float constrainFloat(float value, float min, float max);

bool isTimeoutReached(unsigned long startTime, unsigned long timeoutMs);

void delayWithButtonCheck(unsigned long delayMs);

void resetEncoders();

void driveDistance(int distance, int speed);

void turnAngle(int angle, int speed);

void driveTicks(int ticks, int speed);

void turnRight90();

void turnLeft90();

void turn180();

================================================================================

FILE 8: Utils.cpp - Utility Functions Implementation

================================================================================

#include "Utils.h"

#include "Hardware.h"

#include "Config.h"

void waitForButtonPress() {

display.gotoXY(0, 3);

display.print("Press any btn");

while (!buttonA.isPressed() && !buttonB.isPressed() && !buttonC.isPressed()) {

delay(10);

}

while (buttonA.isPressed() || buttonB.isPressed() || buttonC.isPressed()) {

delay(10);

}

}

void calibrateLineSensors() {

display.clear();

display.print("Calibrating");

display.gotoXY(0, 1);

display.print("Line Sensors");

lineSensors.initFiveSensors();

motors.setSpeeds(150, -150);

for (int i = 0; i < CALIBRATION_SPINS; i++) {

lineSensors.calibrate();

delay(CALIBRATION_DELAY_MS);

}

motors.setSpeeds(0, 0);

display.gotoXY(0, 2);

display.print("Complete!");

playSuccessSound();

delay(500);

}

void calibrateProximitySensors() {

display.clear();

display.print("Calibrating");

display.gotoXY(0, 1);

display.print("Prox Sensors");

prox.initThreeSensors();

display.gotoXY(0, 2);

display.print("Complete!");

playSuccessSound();

delay(500);

}

void showSensorReadings() {

lineSensors.read(lineValues);

prox.read();

display.clear();

display.print("L:");

display.print(lineValues[0]);

display.print(" ");

display.print(lineValues[1]);

display.gotoXY(0, 1);

display.print("M:");

display.print(lineValues[2]);

display.gotoXY(0, 2);

display.print("R:");

display.print(lineValues[3]);

display.print(" ");

display.print(lineValues[4]);

display.gotoXY(0, 3);

display.print("P:");

display.print(prox.countsLeftWithLeftLeds());

display.print(" ");

display.print(prox.countsFrontWithLeftLeds());

display.print(" ");

display.print(prox.countsRightWithRightLeds());

}

void performSystemCheck() {

display.clear();

display.print("System Check");

display.gotoXY(0, 1);

display.print("Motors...");

motors.setSpeeds(100, 100);

delay(500);

motors.setSpeeds(-100, -100);

delay(500);

motors.setSpeeds(0, 0);

display.gotoXY(0, 2);

display.print("Buzzer...");

buzzer.play("c32");

delay(500);

display.gotoXY(0, 3);

display.print("Complete!");

playSuccessSound();

delay(1000);

}

int mapRange(int value, int fromLow, int fromHigh, int toLow, int toHigh) {

return (value - fromLow) * (toHigh - toLow) / (fromHigh - fromLow) + toLow;

}

float constrainFloat(float value, float min, float max) {

if (value < min) return min;

if (value > max) return max;

return value;

}

bool isTimeoutReached(unsigned long startTime, unsigned long timeoutMs) {

return (millis() - startTime) > timeoutMs;

}

void delayWithButtonCheck(unsigned long delayMs) {

unsigned long startTime = millis();

while (!isTimeoutReached(startTime, delayMs)) {

if (buttonB.isPressed()) break;

delay(10);

}

}

void resetEncoders() {

encoders.getCountsAndResetLeft();

encoders.getCountsAndResetRight();

}

void driveTicks(int ticks, int speed) {

resetEncoders();

while (encoders.getCountsLeft() < ticks && encoders.getCountsRight() < ticks) {

motors.setSpeeds(speed, speed);

if (buttonB.isPressed()) break;

}

motors.setSpeeds(0, 0);

delay(200);

}

void turnRight90() {

resetEncoders();

while (abs(encoders.getCountsLeft()) < TURN_90_TICKS) {

motors.setSpeeds(TURN_SPEED, -TURN_SPEED);

if (buttonB.isPressed()) break;

}

motors.setSpeeds(0, 0);

delay(200);

}

void turnLeft90() {

resetEncoders();

while (abs(encoders.getCountsLeft()) < TURN_90_TICKS) {

motors.setSpeeds(-TURN_SPEED, TURN_SPEED);

if (buttonB.isPressed()) break;

}

motors.setSpeeds(0, 0);

delay(200);

}

void turn180() {

resetEncoders();

while (abs(encoders.getCountsLeft()) < TURN_180_TICKS) {

motors.setSpeeds(TURN_SPEED, -TURN_SPEED);

if (buttonB.isPressed()) break;

}

motors.setSpeeds(0, 0);

delay(200);

}

================================================================================

FILE 9: Menu.h - Menu System Declarations

================================================================================

#pragma once

#include "Modes.h"

const char* getModeName(Mode mode);

void showMenu();

Mode selectMode();

void displayModeInfo(Mode mode);

================================================================================

FILE 10: Menu.cpp - Menu System Implementation

================================================================================

#include "Menu.h"

#include "Hardware.h"

#include "Config.h"

static const char* modeNames[] = {

"Spiral",

"Sq Spiral",

"Zig-Zag",

"Sq Zig-Zag",

"Circle"

};

const char* getModeName(Mode mode) {

if (mode >= 0 && mode < MODE_COUNT) {

return modeNames[mode];

}

return "Unknown";

}

void showMenu() {

display.clear();

display.print("A/C:Chg B:Run");

}

Mode selectMode() {

static Mode currentMode = MODE_SQUARE_SPIRAL;

showMenu();

display.gotoXY(0, 1);

display.print(getModeName(currentMode));

delay(100);

Mode lastDisplayedMode = currentMode;

while (true) {

if (currentMode != lastDisplayedMode) {

display.gotoXY(0, 1);

display.print("                ");

display.gotoXY(0, 1);

display.print(getModeName(currentMode));

lastDisplayedMode = currentMode;

}

if (buttonA.getSingleDebouncedPress()) {

currentMode = (Mode)((currentMode + 1) % MODE_COUNT);

playModeChangeSound();

}

if (buttonC.getSingleDebouncedPress()) {

currentMode = (Mode)((currentMode - 1 + MODE_COUNT) % MODE_COUNT);

playModeChangeSound();

}

if (buttonB.getSingleDebouncedPress()) {

displayModeInfo(currentMode);

return currentMode;

}

delay(50);

}

}

void displayModeInfo(Mode mode) {

display.clear();

display.print("Starting:");

display.gotoXY(0, 1);

display.print(getModeName(mode));

display.gotoXY(0, 2);

display.print("Press B=Stop");

delay(1000);

}

================================================================================

END OF CODE SUBMISSION

================================================================================

GitHub Repository: https://github.com/bhanuGupta1/zumo-os-project

# Marking rubric

Driving

| Driving Behaviour | Perfect (3 marks) | Mostly on track (2 marks) | Roughly on track (1 mark) | None (0 marks) |
| --- | --- | --- | --- | --- |
| Square Spiral (3 Marks) | Clean 90° turns, straight lines, even spacing throughout. Sharp corners, no drift. | Recognizable spiral with mostly 90° turns. Minor deviations in straightness or spacing. | Attempts spiral but with significant turn or line issues. Barely recognizable square shape. | No recognizable spiral pattern or basic control. Turns not 90°. |
| Zig-zag (3 Marks) | Consistent angles and segment lengths. Smooth, uniform turns. Symmetrical pattern throughout. | Recognizable zig-zag with generally consistent angles. Some variation but clear back-and-forth motion. | Attempts zig-zag but angles vary widely or segments very uneven. Identifiable but poor execution. | Cannot maintain zig-zag pattern. Random wandering, no discernible pattern. |
| Square Zig-zag (3 Marks) | Straight parallel lines, precise 90° turns. Even spacing, no gaps/overlaps. Complete coverage. | Recognizable lawn-mowing pattern with parallel lines. Minor issues with straightness or spacing. | Attempts area coverage but lines not parallel, spacing uneven, or areas missed. Intent visible. | Cannot execute area-coverage pattern. Random movement, no systematic structure. |
| Perfect Circle (3 Marks) | Smooth continuous circle, constant radius. Endpoint meets startpoint (±5cm). Not a spiral. | Generally circular path that closes well (±10cm). Minor radius variation but clearly a circle, not spiral. | Attempts circle but with flat sections, irregular radius, or poor closure (10-20cm gap). | Cannot maintain circular path. Spirals, very irregular shape, or fails to close. |

Software

| Task ↓ | Worthy of top marks 9 – 7 marks | Honest attempt to be better than average 6 – 5 marks | Struggling 4 – 3 marks | Insufficient 2 – 1 marks |
| --- | --- | --- | --- | --- |
| Software: (9 Marks) | - The code is well structured and adheres to recognised best practices. - The methods implemented are efficient, with no unnecessary or nested loops that reduce readability. - Appropriate comments are included to enhance code understanding and maintainability. | - The code is generally acceptable, though it shows occasional lack of organisation; however, a clear effort towards maintaining structure is evident. - Appropriate methods are mostly employed, and the loop() function demonstrates a tidy and logical implementation. | - The code lacks structure and organisation, though it functions to a limited extent. - Most control logic is concentrated within the loop() function, with minimal use of modular methods. | - Code not fit for purpose, does not work. - None or few methods used, used poorly or is non-functional. |
| Viva voce examination (9 Marks) | - The response demonstrates full and confident understanding of the program, strongly indicating it was written by the student. | - The response shows clear understanding of the program’s structure and logic, indicating the student likely wrote it. | - The response shows limited understanding, with signs that only parts of the code were written or understood by the student. | - The response shows no understanding of the program, suggesting it was not written by the student. |